# CONTROL PLAN

Template CP-STD-01 Rev 2

Part number: {{PART_NO}}    Rev: {{REV}}    Prepared: {{DATE}}

| Process step | Characteristic | Specification | Control method | Reaction plan |
| --- | --- | --- | --- | --- |
| 20 Machine profile | Datum A flatness | 0.05 | CMM, first piece + 1/10 | Stop, re-fixture, re-inspect |
| 30 Tap | Thread size | Per callout | GO/NOGO gage 100% | Segregate, rework if possible |
| 50 Finish | Anodize thickness | Per QS-114 Rev B | Supplier cert per lot | Reject lot, notify supplier |
| 60 Final | All characteristics | Per inspection plan | Per IP frequency | Hold, MRB |